LfcBpr User Guide

Version Number: 2.02

Date of Issue: 2021/6/21

Sea Zhou

dahai.zhou@lcfuturecenter.com

Cissie Gu

feng.gu@lcfuturecenter.com

The spec “Owner” is responsible for identification, maintenance, and publication of the controlled document.

Revision History

| Date of Issue | Version | Owner | Revision Comments |
| --- | --- | --- | --- |
| 2020/07/28 | 0.01 | Sea Zhou Cissie Gu | - Draft version |
| 2020/08/02 | 0.02 | Sea Zhou | - Add 2.5 Common |
| 2021/04/12 | 0.03 | Cissie Gu | - Add that continue from the break. |
| 2021/04/29 | 2.00 | Cissie Gu | - Remove ReadMe.txt, add LfcRelaseNote.txt to modify |
| 2021/06/18 | 2.01 | Cissie Gu | - Add new items in PorjectConfig.ini - Can modify xx.ini and xx.rsp by projects - Support run b_xx.bat first - Add clean build |
| 2021/06/21 | 2.02 | Cissie Gu | - Add MFG binaries for several BIOS ROMs |

Table of contents

1	Introduction	4

2	Preparatory	4

2.1	Insyde	4

2.2	Phoenix	4

2.3	ProjectConfig.ini	5

2.4	ReleaseNote.txt	5

2.5	Common	5

3	Run	6

3.1	Command Line:	6

3.2	GUI	7

4	Additional Information	8

Introduction

The LfcBpr is a tool to automatically build, sign and pack BIOS release package, it used on LNB projects, support Insyde and Phoenix codebase. This is the user guide to LfcBpr tool. LfcBpr stands for LCFC Feature Code Build Pack Release.

Preparatory

Insyde

- Get Insyde codebase with L05, L05 is a MUST.

- Make Insyde build pass, for example: nmake uefi64

- Merge Intel ME by yourself, this normally in ProjectPostBuild.bat, here is an example:

@echo ------------Pack ME------------

@if not defined PLFM_NAME      set PLFM_NAME=TigerLake

@if not defined ME_VERSION     set ME_VERSION=15.0.0.1224

@if not defined ME_BIN_NAME    set ME_BIN_NAME=CSME_B0_Consumer_15.0.0.1224_prod

@if not defined PMC_BIN_NAME   set PMC_BIN_NAME=PMC_B0_150.01.20.1009_prod

@if not defined TCSS_IOM       set TCSS_IOM=IOM_B0_11.0008.0.0_prod

@if not defined TCSS_MGPHY     set TCSS_MGPHY=PHY_B0_2.2022_prod

@if not defined TCSS_TBT       set TCSS_TBT=TBT_B0_Rev27_preprod

cd ME\%ME_VERSION%

@fit.exe -b -f %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\ME\%ME_VERSION%\S540.xml -me %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\ME\%ME_VERSION%\CSME\Silicon\LP\%ME_BIN_NAME%.bin -bios %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\BIOS\%PLFM_NAME%.fd -pmcp %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\ME\%ME_VERSION%\PMC\%PMC_BIN_NAME%.bin -iom %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\ME\%ME_VERSION%\TCSS\IOM\%TCSS_IOM%.bin  -nphy %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\ME\%ME_VERSION%\TCSS\MGPHY\%TCSS_MGPHY%.bin -tbt %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\ME\%ME_VERSION%\TCSS\TBTYFL\%TCSS_TBT%.BIN -o %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\bios_smt.bin

- Ensure the final BIOS output file name is bios_smt.bin, file location MUST be in the project folder, for example: Board\Oem\L05TigerLakeMultiBoardPkg\ bios_smt.bin. The bios_smt.bin is 16M or 32M raw BIOS binary file for SF100.

- Ensure the final crisis output file name is crisis.bin, file location MUST be in the project folder, for example: Board\Oem\L05TigerLakeMultiBoardPkg\crisis.bin. Here is an example:

@copy %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\BIOS\%PLFM_NAME%.fd %WORKSPACE%\%PROJECT_REL_PATH%\%PROJECT_PKG%\crisis.bin

- Add the project_XX.uni file for your BIOS build ID, normally copy it from project.uni. For example, your build ID is F5, then copy project.uni to project_F5.uni

- If have binaries for flashing BIOS ROMs respectively, need put the binaries to the project folder as bios_smt1.bin, bios_smt2.bin and so on.

Phoenix

- Get Phoenix codebase with OIL, OIL is a MUST

- Make Phoenix build pass, for example: PhMake.bat

- Ensure the final BIOS output file name is bios_smt.bin, file location MUST be in the project folder, for example: Phoenix\Projects\Celadon\000\bios_smt.bin. The bios_smt.bin is 16M or 32M raw BIOS binary file for SF100. Here is an example:

copy  "$(BUILD_BIN_DIR)\$(PLATFORM_NAME)X64.bin" "$(BIOS_PATH)\bios_smt.bin"

- Ensure the final BIOS capsule file output file name is bios_cap.cap, file location MUST be in the project folder, for example: Phoenix\Projects\Celadon\000\bios_cap.cap. The bios_cap.cap is 17.x M or 33.x M BIOS capsule file made by UCWW.exe, could be used for Secure Flash and Secure Crisis, this normally in project.mak, here is an example:

@cd "$(BUILD_DIR)\FV\SecureFlash"

UCWW.exe \

-buildcap rsa2048 4 9166175-35F4-4A03-B45B-EF6FC908468A \

-bios "$(BUILD_BIN_DIR)\$(PLATFORM_NAME)X64.bin" \

-ufa "$(WORKSPACE)\$(SCT_MODULE_SYSTEM)\PhSystemBinPkg\FlashUpdateDriver\X64\SystemFlashUpdateDriverDxe.efi" \

-out bios_cap.cap

- Add the BIOS_Version_XX.def file for your BIOS build ID and be included by project_XX.def. The BIOS_Version_XX.def must contain the BIOS version string, for example:

Config BIOS_VERSION, "\"F0CN44WW\""    // set the revision of BIOS.

- If have binaries for flashing BIOS ROMs respectively, need put the binaries to the project folder as bios_smt1.bin, bios_smt2.bin and so on.

LfcBprInput

Copy the Tool\LfcBprInput into project folder and config it CAREFULLY

ProjectConfig.ini

Config it for your project.

If you want to upload formal release bios to ftp, please fill in the FtpPath

If you want to send formal release bios mail , please fill in the ProjectMailGroup

xx.rsp and xx.ini

xx need to be moidied as your project BuildId

| Insyde | Purpose |
| --- | --- |
| Platform_NAC_ xx.ini | Build test release bios exe |
| Platform_MFG_ xx.ini | Build formal release MFG bios exe |
| Platform_REL_ xx.ini | Build formal release windows exe |
| Phoenix |  |
| default_NAC _xx.rsp | Build test bios exe |
| default_MFG _xx.rsp | Build formal release MFG bios exe |
| default_REL _xx.rsp | Build formal release windows exe |

- 2.3.2ReleaseNote.txt

Formal release package contain ReadMe.txt and Web_Release\Readme.docx, tool will not auto generate them, it must be human's work, so you must copy the Tool\TemplateLnb\LfcReleaseNote_XX.txt in your project folder,and modify what in the red rectangle, the format is showed as below:

Flash Note means if you has something to explain to user how to flash BIOS,

You can modify the N/A ,this text will be showed in formal release mail.

Tool will read the WHOLE FILE and put it into ReleaseNote.txt and extract to ReadMe .txt and Web_Release\Readme.docx.

Common

- Must install Mocca key for Mocca sign (local sign)

- Outlook must be installed and opened for LENOVO sign

Run

This tool no need to install, copy it to anywhere on your disk then run.

It has FIVE parameters to input:

| PARAMETERS | MUST? | DESCRIPTION |
| --- | --- | --- |
| Build ID | Mandatory | XX is normal build XXC is build clean XX1, XX2, XX3 please see the help message |
| BIOS Version | Optional | If not input, will use the version in BIOS code, it input, will use it to override BIOS code 10: major version 10, for example: A7CN10WW 10TTTTTT: major version 10 + test version TTTTTT, for example: A7CN10WWTTTTTT |
| Sign Type | Optional | If not input, default sign type is Mocca sign (local sign) If input, sign type could be Mocca sign (local sign) or LENOVO sign (server sign) |
| Release Type | Optional | If not input, default release type is test release If input, release type could be test release or formal release |
| Project Folder | Optional | The path to project folder If not input, will use the tool folder as project folder |

Command Line:

b.bat /? will show help message as below:

| PARAMETERS | MUST? | USAGE |
| --- | --- | --- |
| Build ID | Mandatory | b.bat XX /p "F:\Project\TGL\Tag0014\Board\Oem\L05TigerLakeMultiBoardPkg" |
| BIOS Version | Optional | b.bat XX VV /p "F:\Project\TGL\Tag0014\Board\Oem\L05TigerLakeMultiBoardPkg" |
| Sign Type | Optional | b.bat XX /m or /l /p "F:\Project\TGL\Tag0014\Board\Oem\L05TigerLakeMultiBoardPkg" |
| Release Type | Optional | b.bat XX /t or /f /p "F:\Project\TGL\Tag0014\Board\Oem\L05TigerLakeMultiBoardPkg" |
| Clean build or not(if type in /f will clean build automatically whatever has /c or not) | Optional | b.bat XX /t /c or /f /c /p "F:\Project\TGL\Tag0014\Board\Oem\L05TigerLakeMultiBoardPkg" |
| Project Folder | Optional | b.bat XX /p "F:\Project\TGL\Tag0014\Board\Oem\L05TigerLakeMultiBoardPkg" |

Because the tool can support any source code in your hard disk, so you must input the /p with source code path. If you only have one source code to build, you can choose one option below to remove the usage of /p:

- Copy the LfcBpr folder to your project folder

For example, copy the whole folder to project folder Board\Oem\L05TigerLakeMultiBoardPkg (Insyde) or Phoenix\Projects\Celadon\000 (Phoenix)

- Modify the b.bat

BEFORE:

@Tool\Python38\python.exe LfcBpr.py %1 %2 %3 %4 %5 %6

AFTER:

@Tool\Python38\python.exe LfcBpr.py /p "F:\Project\TGL\Tag0014\Board\Oem\L05TigerLakeMultiBoardPkg" %1 %2 %3 %4 %5 %6

GUI

| PARAMETERS | MUST? | USAGE |
| --- | --- | --- |
| Build ID | Mandatory | See picture Note: If you need to continue to build from break, you can use BuildID: A to build it again. |
| BIOS Version | Optional | See picture |
| Sign Type | Optional | See picture |
| Release Type | Optional | See picture |
| Project Folder | Optional | See picture |

Build debug bios

You can click help to check how to build.

Get the bios package

If build success, you can click open to get build package

Continuous build from break note

Most of all, we face that the server cannot reply or give the signed BIOS slowly. At this time, we often sign the input.7z manually, then we can use the BUILD ID A to go on.

- Put the signed package into the LfcBprOutput\xxx\06_wait_for_sign_back\. XXXX must be the newest document under the LfcBprOutput

2.   Copy and override the green marked part to the LfcBprOutput\xxx\data.json and modify the address as your own project.

"06_wait_for_sign_back_output": {

"file": "LfcBprOutput\\2020-12-25-17-52-44-GZCN07WW\\06_wait_for_sign_back\\Signed.7z"

},

"06_wait_for_sign_back_result": 0,

"07_tool_7z_uncompress1_output": {

"file": "LfcBprOutput\\2020-12-25-17-52-44-GZCN07WW\\07_tool_7z_uncompress1\\Unzipped1"

}

- Open LfcBpr, type A  in the BUILD ID input box then build.

Additional Information

- Tool will copy all aaaaaa_XX.bbb in the project folder to aaaaaa.bbb, so you can create Project_XX.env, ProjectPostBuild_XX.bat, Project_XX.var, project_XX.def if necessary

- Tool will run b_xx.bat if has in the project folder, or skip it.

- Tool will always build FSP for Intel platform